# FIND-001 - Availability flaps during recovery from a network partition

| Related test | TC-NF-004 |
| --- | --- |
| Severity | Low-Medium |
| Type | Recovery / consistency |

# Summary

When a partitioned node reconnects, availability does not recover in a single clean transition; the probe recorded two outage windows before it stabilised.

# Expected

One outage window - down at disconnect, up once reconnected.

# Actual

Two outage windows; a short second dip after the first recovery.

# Impact

A client treating the first success as 'recovered' may hit a second brief failure.

# Recommendation

Define recovery as sustained availability, not the first success.

# Steps to reproduce

Load the sample graph; confirm HTTP 200.

Isolate tg3 from the network.

Probe ping_count every 250 ms.

Reconnect tg3 and keep probing.